# Appendix A - Minimum Measurement & Selection Record

Controlled decision gate for the provisional envelope and speaking length

DISPOSITION: HOLD. This appendix defines evidence sufficient to decide whether the 42 x 16 in envelope and 32 in speaking length may be retained. It does not presume that any unresolved item passes.

# A1. Purpose and scope

Rev A §11 permits retention only after the actual vessel and selected strings clear the stated limits. Section §4 adds room, tuner, spine, restraint, top-plate, center-of-gravity, target-pitch, and SPL facts that must be recorded before their dependent actions. The smallest defensible record therefore separates selection, cutting, tensioning, and commissioning instead of treating them as one approval.

# A2. Decision hierarchy

| Gate | Question answered | Minimum evidence | Effect of failure or unknown |
| --- | --- | --- | --- |
| G0 - Record complete | Are the selected physical items and measurement methods identifiable? | Date, measurer, item IDs, units, method/source, actual values, status | HOLD; no inference from nominal dimensions |
| G1 - Envelope | Can the 42 in height and 16 in base coexist with room, vessel, plate, access, and clearances? | Room envelope; vessel geometry/condition/mass; plate geometry; layout clearances | Resize around vessel; repeat affected geometry |
| G2 - 32 in scale | Can five selected strings operate at target pitches within the Rev A cap and tuner/load-path ratings? | Per-string identity, pitch, exact-scale tension, source; tuner and load-path ratings | Change string/pitch/scale; recalculate all five |
| G3 - Cutting | Are G1 and the dimensional portions of G2 fully passed? | Signed worksheet with no applicable fail/unknown; cut plan revision identified | No cutting |
| G4 - Tensioning | Is the built load path restrained, guarded, rated, inspected, and ready for staged loading? | Known ratings; dry-fit; restraint; guards; T0 checks; staged plan | No tensioning |
| G5 - Playing/tests | Have static proof, tip test, and SPL control passed? | T1/T2 results; numeric SPL ceiling and meter setup | No playing, sweep, or powered excitation |

# A3. Governing retain criteria

| Candidate feature | Pass only when documented | Not sufficient |
| --- | --- | --- |
| 42 x 16 in envelope | Clear room height and declared walking/service clearance fit; vessel max OD <=14 in and clear ID >=12 in; intended string/hardware clearance >=1 in; plate 14-18 in and safely mountable; independent vessel restraint fits. | “Looks like it fits,” nominal vessel size, or room dimensions without access clearance. |
| 32 in speaking length | Five equal speaking lengths can be laid out within 1/16 in; spacing is 1.5-2.25 in; string-to-spine clearance >=1.5 in; each exact selected string at exact pitch and 32 in scale is <=20 lbf; sum <=100 lbf; safe tuning range and source are recorded. | Gauge alone, approximate pitch, a generic calculator without string construction/unit weight, or a total that hides one over-limit string. |
| Selected tuner/load path | Each tuner rating meets its string tension; spine, anchors, fasteners, base connection, and head block have known allowable ratings not below applied demand; anchor safety factor is at least 4x. | Hardware appearance, seller category (“bass tuner”), or ungraded fasteners. |

# A4. Required record fields

Use the companion worksheet. A valid entry contains all six elements: item identity; measured/calculated value; unit; method or authoritative source; acceptance limit; and pass/fail/unknown result. Photographs and source sheets should carry the same record ID.

# A5. Items that do not decide the cut envelope but cannot be erased

Restraint, estimated assembly mass/center of gravity, and permitted SPL are downstream controls. They may remain HOLD while G1/G2 are evaluated, but they must not be misrepresented as passed. Tensioning requires a structural stud, restraint hardware rated at least 200 lbf and at least four times its calculated design demand, complete load-path ratings, guards, dry-fit inspection, and staged loading. Playing requires the actual 10-degree tip/restraint test and static proof. Acoustic testing requires a numeric SPL ceiling, weighting/time response, meter, and operator measurement position.

# A6. Decision rule and change control

| Recorded outcome | Permitted disposition |
| --- | --- |
| Every G1 criterion passes; every G2 criterion passes | 42 x 16 in and 32 in may be retained for the identified selections. Cutting may be marked eligible only after the cut plan matches those selections. |
| G1 passes; G2 fails/unknown | Envelope may be retained provisionally; no anchor-bar cutting or tensioning until string/scale selection is recalculated and passed. |
| G1 fails/unknown | Keep HOLD. Resize around the actual vessel/room and repeat all affected clearances before cutting. |
| Any rating is unknown | No tensioning. A completed dimensional record does not substitute for a rating. |
| Any selected component, pitch, scale, or room location changes | Void affected rows; issue a new record revision; recalculate before continuing. |

NO AUTOMATIC CLEARANCE. Completing the form produces a decision record, not permission. Only actual documented passes unlock the corresponding next gate; all other states remain HOLD.

# A7. Record retention

Retain the signed worksheet, photographs, raw measurements, manufacturer specifications, calculator output, cut-plan revision, and later T0-T2/SPL logs under one record ID. Preserve failed and superseded records; do not overwrite them.